LOST IN SPACE TRACK

Hackathon Problem Statement  ·  Earth-Observation Attitude & Imaging Scheduling

| Track | Lost in Space — Autonomous EO Tasking |
| --- | --- |
| Deliverable | One Python file exporting plan_imaging(...) + a 2-minute presentation |
| Test cases | 3 orbits (0°, 30°, 60° cross-track offset from AOI) |
| Judging | Presentations first, then code verification for the top 3 |

# 1. Scenario

A small Earth-observation satellite is in a low Earth orbit and will pass near a fixed rectangular Area of Interest (AOI) on the ground. The satellite carries a single staring imager fixed to the +Z body axis. The only way to aim the imager is to slew the spacecraft body using its reaction wheels. Participants must design a slew and imaging scheduling algorithm that decides what to image, when, and in which body attitude during each pass — subject to the physical limits of the ACS.

Your algorithm will be wrapped into a Basilisk spacecraft simulation on the organizers' side and executed over three pre-defined passes (TLEs in Section 8). The simulator produces the score automatically. See Section 9 for the judging flow.

# 2. Spacecraft Specifications

| Parameter | Value |
| --- | --- |
| Imager boresight | +Z body axis |
| Integration time per image | 120 ms (ground footprint must be stable during this window) |
| Reaction wheels | 4 wheels in a 45° pyramidal configuration (cant = 45° from +Z; azimuths 0°, 90°, 180°, 270°) |
| Per-wheel momentum capacity | 30 mNms |
| Body momentum envelope (X, Y) | ≈ √2 · 30 = 42.4 mNms |
| Body momentum envelope (Z) | ≈ 2√2 · 30 = 84.9 mNms |
| Spacecraft inertia (assumed) | diag(0.12, 0.12, 0.08) kg·m² |
| Orbit altitude (nominal) | ≈ 500 km, sun-synchronous |
| Imager FOV (assumed) | 2.0° × 2.0° (≈ 17 × 17 km at nadir from 500 km) |

Slew dynamics follow H_body = I·ω + H_wheels. Wheels are momentum-limited (no desaturation thrusters during a pass). Treat inertia and wheel layout as fixed; do not attempt to modify the plant in the simulator.

# 3. Operational Constraints

- Smear constraint during integration: body rate magnitude |ω_body| ≤ 0.05°/s continuously for the full 120 ms window of any valid image.

- Off-nadir limit: maximum pointing angle from nadir ≤ 60°.

- Wheel saturation: at no time may any single wheel exceed its 30 mNms momentum limit.

- Imager FOV: fixed at 2.0° × 2.0° for all teams and all three test cases.

- Pass window: each test case is a 12-minute window bracketing closest approach of the satellite to the AOI (see Section 8).

# 4. The Task

Design and implement a Python function that, given a propagated ephemeris (from the supplied TLE) and the AOI geometry, produces a time-tagged attitude and imaging schedule. Your function will be called by the organizers' Basilisk harness; the exact interface is specified in Section 7.

Teams are free to choose what they optimize. Examples:

- Maximize AOI coverage (greedy or globally-planned mosaicking).

- Minimize control effort (∫|τ·ω| dt, or total Δh across wheels).

- Minimize mission time used within the pass.

- Robustness-weighted coverage (same plan performs well across all three geometries).

Explain your chosen objective and strategy in your 2-minute presentation. You can also tag it in the schedule["objective"] field for the record (Section 7.3), but the presentation is what the judges score.

# 5. Scoring Metric (Automated)

Each of the three test orbits is scored independently by the Basilisk harness, then combined. Per-orbit score:

S_orbit  =  C · (1 + α·η_E + β·η_T) · Q_smear

| Symbol | Meaning |
| --- | --- |
| C | Fraction of AOI area imaged with valid, non-smeared frames (0–1). |
| η_E | Control-effort efficiency = 1 − ΔH_used / ΔH_budget, clipped to [0,1]. ΔH_budget = 200 mNms per pass. |
| η_T | Time efficiency = 1 − T_active / T_pass, where T_active is cumulative slew+image time. |
| Q_smear | Smear quality factor: 1.0 if all kept frames meet the rate constraint; linearly penalized down to 0 based on fraction of frames violating it. |
| α, β | Reward weights: α = 0.25, β = 0.10. |

Total score across all three test cases:

S_total  =  0.25 · S_case1  +  0.35 · S_case2  +  0.40 · S_case3

Case 3 (60° offset) is the hardest and is weighted highest.

Hard disqualifiers for a frame (frame is discarded and counts as non-imaged area): (i) wheel saturation, (ii) off-nadir > 60°, (iii) smear rate exceeded during the 120 ms integration.

# 6. Area of Interest

A rectangular AOI in the Po Valley, northern Italy (chosen for a convenient mid-latitude SSO pass geometry). The same AOI is used for all three test cases — only the satellite ground-track offset changes.

| Corner | Latitude | Longitude |
| --- | --- | --- |
| SW | 44.55° N | 9.37° E |
| SE | 44.55° N | 10.63° E |
| NE | 45.45° N | 10.63° E |
| NW | 45.45° N | 9.37° E |
| Center (reference) | 45.00° N | 10.00° E |

Approximate dimensions: ~100 km (E–W) × ~100 km (N–S) ≈ 10,000 km². The AOI is supplied to your function as a polygon (see Section 7).

# 7. What You Submit

You submit exactly one Python file that exports a function named plan_imaging(...) with the signature in Section 7.1. Name the file anything you like — the grader imports the function by name. No zip, no folder structure, no accompanying documents.

The organizers run a Basilisk-based simulation on their end. The grader imports your file, calls plan_imaging once per test case, and passes the returned schedule to a Basilisk attitude controller that tracks it.

## 7.1  Entry-point signature

Your file must export exactly this function, with exactly this signature (the grader imports it by name):

# Any filename; exports plan_imaging(...) at module top level.

# Pre-installed dependencies: numpy, scipy, sgp4. Do not rely on anything else.

from typing import List, Dict, Any, Tuple

def plan_imaging(tle_line1:   str,

tle_line2:   str,

aoi_polygon_llh: List[Tuple[float, float]],

pass_start_utc:  str,

pass_end_utc:    str,

sc_params:       Dict[str, Any]

) -> Dict[str, Any]:

"""

Compute an attitude + imaging schedule for ONE pass.

Parameters

----------

tle_line1, tle_line2 : str

Standard two-line element set. Propagate with SGP4

(sgp4.api.Satrec.twoline2rv) in the TEME frame, then rotate to

ECI/J2000 before reasoning about attitude.

aoi_polygon_llh : list of (lat_deg, lon_deg)

Closed polygon defining the AOI on the WGS-84 ellipsoid. The first

and last vertices are equal.

pass_start_utc, pass_end_utc : str

ISO-8601 Zulu timestamps bounding the scored pass window

(e.g. "2026-04-23T17:24:00Z"). All 't' values in your returned

schedule are SECONDS relative to pass_start_utc.

sc_params : dict

Spacecraft parameters -- see Section 7.2.

Returns

-------

schedule : dict

See Section 7.3 for the exact required structure.

"""

...

return schedule

## 7.2  sc_params dictionary (passed in by the grader)

sc_params = {

"inertia_kgm2":   [[0.12, 0.0,  0.0 ],

[0.0,  0.12, 0.0 ],

[0.0,  0.0,  0.08]],

"wheel_layout":   "pyramid_45deg",      # 4 RWs, 45° cant, azimuths 0/90/180/270

"wheel_Hmax_Nms": 0.030,                # 30 mNms per wheel

"n_wheels":       4,

"integration_s":        0.120,          # shutter dwell -- IMMUTABLE

"fov_deg":              [2.0, 2.0],     # cross-track, along-track

"imager_boresight_B":   [0.0, 0.0, 1.0], # +Z in body frame

"smear_rate_limit_dps": 0.05,           # deg/s, on |omega_body|

"off_nadir_max_deg":    60.0,

"earth_model":    "WGS84",

"eci_frame":      "J2000",

}

## 7.3  Return value: the schedule dict

Your function must return a dictionary with exactly the following keys. The grader validates structure before the simulation runs; malformed schedules fail the test case with S_orbit = 0.

schedule = {

# --- 1. Declared objective (string, diagnostic only) ------------

"objective": "max_coverage",

# --- 2. Attitude command trajectory -----------------------------

# Ordered list, strictly monotonic in 't' (seconds from

# pass_start_utc). The grader SLERPs between samples.

#

# Quaternion convention: q_BN maps BODY -> INERTIAL (ECI, J2000),

# SCALAR-LAST ordering  [qx, qy, qz, qw], unit-norm.

# Rule of thumb: 10-50 Hz; minimum 20 ms between samples.

# First sample MUST be at t = 0.0. Last sample MUST be at or after

# the last shutter window's end time.

"attitude": [

{"t": 0.000,  "q_BN": [0.0, 0.0, 0.0, 1.0]},

{"t": 0.020,  "q_BN": [qx,  qy,  qz,  qw ]},

# ...

{"t": T_end,  "q_BN": [qx,  qy,  qz,  qw ]},

],

# --- 3. Shutter command list ------------------------------------

# Each window MUST be exactly 0.120 s long. Windows MUST NOT

# overlap and MUST be in ascending t_start order. t_start is

# seconds from pass_start_utc.

"shutter": [

{"t_start": 12.340, "duration": 0.120},

{"t_start": 13.800, "duration": 0.120},

# ...

],

# --- 4. Optional diagnostics (logged, not scored) ---------------

"notes": "Greedy sweep from NW to SE; 1 Hz replan.",

# --- 5. Optional per-frame target hints (logged, not scored) ----

"target_hints_llh": [

{"lat_deg": 45.10, "lon_deg": 9.80},

# ...

],

}

## 7.4  Hard rules on the function

- Single file. One .py file exporting plan_imaging at module top level. No imports from other files you wrote.

- Pure and deterministic. Given the same inputs, return the same schedule. No network calls, no reading files, no wall-clock or randomness without a fixed seed.

- Wall-clock budget: the function must return within 120 seconds per test case. Exceeding the budget fails the case.

- Allowed dependencies: numpy, scipy, sgp4. Nothing else is guaranteed to be present. Do not import Basilisk — the grader owns the simulator.

- Quaternion convention: body-to-inertial (ECI J2000), scalar-last (x, y, z, w), unit-normalised.

- Time base: all t values are seconds from pass_start_utc. First attitude sample at t = 0; last at or after the final shutter window end.

- Shutter windows must be exactly 0.120 s and must not overlap.

- Attitude sample density: ≥ 20 ms spacing, ≤ 50 Hz. The grader will reject schedules with sub-20-ms spacing.

## 7.5  Exact grader call (what runs on the organizers' side)

For full transparency, this is the exact invocation your function will see on the evaluation server.

# organizer-side grader (abridged)

import importlib.util, time

from basilisk_harness import SpacecraftSim, AoiScorer, load_pass_config

def run_one_case(case_id: str, submission_path: str):

cfg = load_pass_config(case_id)

# Import the team's single .py by path and grab plan_imaging.

spec = importlib.util.spec_from_file_location("submission", submission_path)

mod = importlib.util.module_from_spec(spec)

spec.loader.exec_module(mod)

t0 = time.perf_counter()

schedule = mod.plan_imaging(

tle_line1       = cfg.tle1,

tle_line2       = cfg.tle2,

aoi_polygon_llh = cfg.aoi_polygon,

pass_start_utc  = cfg.pass_start,

pass_end_utc    = cfg.pass_end,

sc_params       = cfg.sc_params,

)

assert time.perf_counter() - t0 <= 120.0, "planning budget exceeded"

sim = SpacecraftSim(cfg)

telemetry = sim.run(schedule)

return AoiScorer(cfg).evaluate(schedule, telemetry)

## 7.6  Minimum viable stub (copy-paste starting point)

A schedule that does nothing but satisfies the structural contract — use this as your scaffold. This submission earns S_orbit = 0 because it takes no images, but it passes the structural validator.

# my_submission.py -- minimum viable stub

def plan_imaging(tle_line1, tle_line2, aoi_polygon_llh,

pass_start_utc, pass_end_utc, sc_params):

from datetime import datetime

t0 = datetime.fromisoformat(pass_start_utc.replace("Z", "+00:00"))

t1 = datetime.fromisoformat(pass_end_utc.replace("Z",   "+00:00"))

T  = (t1 - t0).total_seconds()

attitude = [

{"t": 0.0, "q_BN": [0.0, 0.0, 0.0, 1.0]},

{"t": T,   "q_BN": [0.0, 0.0, 0.0, 1.0]},

]

return {

"objective": "custom:stub",

"attitude":  attitude,

"shutter":   [],

"notes":     "structural stub; replace with a real planner",

}

# 8. Test Orbits (TLEs)

Three SSO satellites at ≈ 500 km, inclination 97.4°, differing only in RAAN to create cross-track offsets at the AOI latitude. Common epoch: 2026-04-23 12:00 UTC. Propagate with SGP4. Your plan_imaging function receives both TLE lines directly.

Scored pass window (all three cases): 2026-04-23T17:24:00Z  to  2026-04-23T17:36:00Z — 12 minutes bracketing closest approach.

Case 1 — Direct overpass (ground track through AOI center, ~0° off-nadir):

LOSTINSPACE-A

1 99991U 26001A   26113.50000000  .00000000  00000-0  00000-0 0     7

2 99991  97.4000 296.7000 0001000  90.0000 230.0000 15.21920000    08

Case 2 — ~30° off-nadir (≈ 293 km cross-track offset):

LOSTINSPACE-B

1 99992U 26001B   26113.50000000  .00000000  00000-0  00000-0 0     8

2 99992  97.4000 292.9000 0001000  90.0000 230.0000 15.21920000    07

Case 3 — ~60° off-nadir (≈ 1009 km cross-track offset):

LOSTINSPACE-C

1 99993U 26001C   26113.50000000  .00000000  00000-0  00000-0 0     9

2 99993  97.4000 283.9000 0001000  90.0000 230.0000 15.21920000    08

Note: case 3 places the satellite near the 60° off-nadir limit when aimed at the AOI centroid — some AOI corners may be physically unreachable within the pointing envelope. That is intentional and is why case 3 is weighted highest.

# 9. Deliverables and Judging

You deliver exactly two things:

- One .py file exporting plan_imaging(...) per Section 7.

- A 2-minute presentation explaining your approach: what you optimized for, how your planner works, and what limitations you identified.

## Judging flow

- All teams give their 2-minute presentations.

- Judges select the top 3 based on the quality of approach, clarity of thinking, and plausibility of claimed performance.

- Organizers run the harness on the top-3 submissions to verify the claimed scores match reality. Ties are broken by S_total; further ties by total control effort (lower wins).

- Final ranking is announced based on verified scores.

In short: the presentation gets you into the top 3; the harness confirms you didn't over-promise. If your verified score is significantly below what you claimed in the talk, you'll be re-ranked accordingly.

# 10. Testing Your Submission

Organizers ship a testing kit alongside this document: a Python package containing the structural validator, the official scorer, the three case configs, and a mock physics simulator. You test your file with a single command:

pip install -r requirements.txt

python test_my_submission.py my_submission.py

The mock simulator treats your commanded attitude as perfectly tracked (no controller dynamics or actuator lag). The validator, scorer, gate logic, TLEs, and AOI are identical to the grader — so if your local score is zero, your real score will be zero too.

### Mock vs. real — leave margin

Your local (mock) scores will be optimistic compared to the organizers' Basilisk run. Leave headroom so real-sim controller overshoot doesn't trip the gates:

- Body rate during integration: target ≤ 0.03°/s (limit is 0.05°/s)

- Off-nadir for frames you care about: target ≤ 55° (limit is 60°)

- Never sit wheels at exactly 30 mNms — leave ~5 mNms margin

The kit README walks through the example submissions, including a deliberately-failing nadir-greedy planner that illustrates the classic smear-gate mistake. Read it before you start.

Good luck, and aim well.  ·  Questions: open an issue on the hackathon repository.  ·  Organizers may issue clarifications at any time before the submission deadline.